BRIEF DESCRIPTION OF HOW THE DSI-32 GRAPHICS DEMO WORKS.

When  the  IBM-PC bus was defined no allowance was made  for  any

processor  other than the 8088 to operate upon it.  Thus there is

now  way  that the DSI-32 can directly access (through  hardware)

the  expansion  cards in the PC-BUS.  In  particular,  the  32032

cannot directly write to the PC's screen.  Consequently Definicon

has developed a software protocol whereby the 32032 requests that

the 8088 perform the PC-BUS accesses it needs.  The 8088 can  get

at all the address space of the DSI-32, as well as any other card

in   the  expansion  slots.   32032  software  calls  have   been

implemented for PEEK and POKE,  INP and OUT into the 8088 address

space.  In  particular  a bidirectional block move of memory  has

been implemented.  This allows a block of memory, for example the

size of a color graphics screen image,  to be moved quickly  from

the  32032 to the 8088 (or vice versa).  A complete screen can be

swapped in about 17 milliseconds.  This time is sufficiently fast

so  that the observer sees no screen update,  merely a new  image

appearing  'instantly'.

It  should  be noted that some  graphics  adaptors,  particularly

those  in  the KAYPRO computers,  give a  colored  'confetti'  or

'snow' on the screen whilst the swap is occurring. This is due to

poor  design,  as accesses to the graphics adapter memory are not

being locked out during the display intervals. The only fix is to

get a better color graphics adaptor card.

The  graphics demo implemented in TG.C shows the speed with which

the 32032 can create graphics images and then display them to the

PC operator.

During the first part of the program random rectangles are  being

drawn,  one at a time,  on the screen of the PC. Due to the speed

at  which  the images can be drawn and replaced it  appears  that

many are actually on the screen but there is really only one at a

time.  To  do  this  the TG.C program has set  up  three  virtual

screens in the 32032's memory.  One is blank, and a copy of it is

used  to  blank the other two when  desired.  The  two  remaining

screens are alternately drawn and displayed. As the 8088 accesses

the  32032  memory  using  DMA the update of  the  color  display

adaptor  is  performed  in the background  whilst  the  alternate

screen  is being drawn by the 32032.

The  display adaptor is then switched to the 320 x 200 bit mapped

graphics mode.  The 32032 now draws random circles and lines. The

circle routines use floating point numbers extensively.  The mode

of operation here is somewhat different as now the screen is only

blanked every 10 images (circles or lines).

Finally the 32032 switches the graphics adaptor to the 640 x  200

mode and draws 'high resolution' random circles and lines.

Please  do  not  hesitate to contact Definicon if  you  have  any

further questions about the DSI-32's graphics capabilities...

....enjoy.....

#############