Low-level protocol specification:  IBM to Gateway, Gateway to IBM

The following ASCII codes are used as flags in the manner specified:

242	- Prefix code for sending data codes which are set aside for protocol

use.

243	- Request to send (REQ).

244	- Acknowledge (ACK).

245	- End of packet (END).

A typical data transfer occurs as follows:

IBM wants to send packet to Gateway:	It sends REQ and waits for ACK.

Gateway is ready to recieve packet:	It sends ACK.

IBM sends packet to Gateway followed by END.

The packet itself is encoded so that REQ, ACK, and END never appear in the

text.  This is done by performing the following substitutions:

242 --> 242 0

243 --> 242 1

244 --> 242 2

245 --> 242 3

Note that transfers can occur in both directions simultaneously.  However,

the ACK signal may be imbedded within a data packet and must be

explicitly removed:

IBM wants to send packet to Gateway:	It sends REQ and waits for ACK.

Gateway is ready to recieve packet:	It sends ACK.

IBM starts sending packet.

Gateway wants to send packet to IBM:	It sends REQ and waits for ACK.

IBM is ready to recieve packet:		It sends ACK.

IBM continues sending its packet, while Gateway sends a packet to IBM.

Timeouts may occur if the wait between a REQ and an ACK is too long or

no packet characters are transmitted for too long a time.  In both cases

no recovery action is undertaken:  the other system is assumed to have crashed.

Reciept of protocol codes within a data packet has the following consequences:

REQ	- End portion of data packet being sent has been lost.

ACK	- Should be removed from input packet and its presence logged for

use by the process which is sending characters.

END	- Packet has been completely sent.