term - a H-19 terminal emulator for the IBM PC

This documentation relates some of the quirks in the terminal

emulator. All the source code can be found in /src/ibm/terminal.

1) To exit the emulator type break (this is done by ^Scroll-Lock)

2) To send a ^@ (such as to set the mark in emacs) you have to

actually type cntrl-shift-2. Cntrl-space won't work.

3) Setting the default settings of the terminal is done by

typing  'debug <the name of the emulator>'. When you're in

debug type 'g'. Now your in the emulator just like always.

Get the menu on the screen and set every thing up to the

way you want it to be. Get out of the menu and then the

emulator. You should be back in debug. Type 'w' to save

the changes you have just made and 'q' to exit to dos.

4) The five function keys on the heath terminal are the keys to

the left of the keyboard F1 - F5.

5) The three color keys are

blue - F6

red - F7

gray - F8

6) Setting the baud rate from the menu does not immediatly change

the baud rate. The baud rate is set when the emulator is

run. So setting the baud rate is only useful when you are

using debug to set the default baud rate.

7) The print screen key will still cause the screen to be sent to

the printer, but it will almost definitly look very strange.

This is because the emulator uses a feature of the screen

controller chip to do fast scrolling that the operating system

does not understand about. In order to print the screen using

the print screen feature, then go into the menu temporarily.

This will restore the screen to a state that the operating

system can understand and you can then print the screen.

Known incompatibilities -

The following are not supported:

hold screen

graphics

shifted keypad

alternate keypad

identify as VT-52

x-mit page

x-mit 25th line

ESC x setting of parameters