# WinViewä For Networks

# Application Notes

# Argosy Software, Inc.
RunPC 4.05 For Macintosh

This application note is for informational use only. Citrix makes no representations or warranties with respect to the contents or use of this document or of any of third-party products discussed within.

Citrix Systems, Inc.
210 University Drive
Suite 700
Coral Springs, FL 33071
Phone (305) 755-0559
Fax  (305) 341-6880

Argosy Software, Inc.
1019 Spruce Sheet
Boulder, CO  80302-9898
Phone (303) 444-2100
Fax   (303) 440-6774

## Overview

This application note facilitates the configuration of Argosy's RunPC software and Citrix WinView for Networks.  RunPC takes the place of the Citrix Remote Link client software and allows Macintosh clients to remotely dial into a WinView server via the ICA protocol.  Once connected to the WinView server, the Macintosh client can run text-based DOS and OS/2 applications.

## Disclaimer

The scenarios described in this document have been tested by Citrix.  Other variations to the scenarios described in this document may work, however they have not specifically been tested by Citrix.  In order to recreate the configurations, you should use the specified revision levels of all software products described in this document and stay within the bounds of the features and functions described in this document.

Please note that this application note is a living document and will be modified as new information and versions of the software described herein become available.  Make sure you have the latest version of this document before you begin.  The latest version is always available in the Citrix Forum on Compuserve.

## Requirements

### Argosy software that is required:

Run PC 4.05 or higher (1 disk)

### Client hardware that is required:

A Macintosh running Apple system 6.05 or higher and a High Speed Modem Cable is required if using a 9600bps or greater modem.

## Setup

- Insert the RunPC disk in the Macintosh's disk drive.  Copy the RunPC file to a folder on your Macintosh.

- Start RunPC by double clicking the icon.

- Choose 'Preferences' under the File menu.  Configure your modem for the correct port and baud rate.  Enter in the proper initialization string for your modem.
*	If using a modem from the WinView compatibility list you may be able to use the initialization string supplied with the WinView Remote Link client. A list of the currently supported modems and their initialization strings are provided at the end of this application note.

- Choose 'Dial' from the File menu.  Add an entry with a name and a phone number.

- If desired, a login ID and password may be specified for automatic logins.  Under 'Preferences' in the File menu, choose to configure the 'User Login' option.  Enter a user ID and password valid on the WinView Server and click OK.  A first user program and working directory may also be specified.

## Operation

- Choose 'Dial' from the File menu.  Select entry to dial and click the dial button.  A login prompt should appear after connecting to a WinView server. (Check with your System Administrator to obtain a valid user ID and password.

## Notes

RunPC 4.05 will allow a Macintosh client to connect to a WinView server via the ICA protocol.  Once connected, users can run DOS and OS/2 text based applications.  Remote workstation printing is also supported in this version of RunPC.

Features not currently supported in RunPC are Windows applications and file transfers.

## Problems

There are no known problems at this time.

## Modem List and Initialization strings

### Example:

Modem Name

Inititialization String,

Dialing Prefix String,Dialing Suffix String,Hangup string,Auto Answer string, Baud Rate,Flow Control,Wait for connection (seconds),Connect message

### Apex Data PCMCIA Data/Fax Modem,

AT&F^M~AT W1 X4 &K3^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### AT&T Comsphere 3800 Series Modems,

AT&F^M~AT \\X1 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### AT&T DataPort 14.4/FAX Modem,

AT&F^M~AT &C1 \\X1 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### ATI 14400 ETC-Express PCMCIA Fax-Modem,

AT&F0^M~AT S0=0 &C1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### ATI 14400 ETC-I/E Fax-Modem,

AT&F2^M~AT S0=0 &C1^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Black Box 325 Modem/Modem Card,

ATE1 V1 X4 S0=0 &C1 %B6%C1 \\G\\J\\Q3^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Boca Research M1440 14.4Kbps Modem,

AT&F^M~ATE1V1X4 S0=0 &C1&D1&K3&S1^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Bullet E9696m V.32,

~~~ATE1 V1 X1 &C1 $C1$E1$F4$L3$S1$X5 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Cardinal 9600 V.42bis External Modem,

ATE1 V1 X4 S0=0 &C1&K3&R0 %C1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Compaq PCMCIA 14.4 Data+Fax Modem,

AT&F^M~AT S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### Compaq PCMCIA SPEEDPAQ 144/P Modem,

ATE1 V1 X4 S0=0 &D2&S1 \\G0\\J0\\N3\\Q3\\X1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### Dallas Fax Modem,

AT&F^M~~~ATE1V1X4 &C1&D1&K3&Q9&S1 \\G0\\N3 S0=0^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Evercom 96 Modem,

ATE1 V1 X4 S0=0 &C1&D1&S1 %C1%E1 \\A3\\G0\\J0\\N3\\Q3^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Generic 9600 Baud Modem,

ATE1 &C1 V1 X1 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,9600,60,CONNECT,0,1,

1200,CONNECT 1200^M,2400,CONNECT 2400^M,9600,CONNECT 9600^M,

### GVC 14400bps V.42 Modem,

~~~ATE1 V1 X4 S0=0 &C1&D2 \\N6\\Q3\\J\\V1 %C1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### GVC 28800bps V.42 Fax Modem,

~~~AT&F^M~ATS0=0^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### GVC 9600bps V.42 Modem,

~~~ATE1 V1 X4 S0=0 &C1&D2 \\N6\\Q3\\V1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Hayes Smartmodem OPTIMA 28800 V.FC FAX,

AT&F^M~AT W1 &C1&Q9&S2 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Hayes Smartmodem OPTIMA 96/144,

ATE1 V1 X4 N4 W1 &C1&K3&Q9&S2 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Hayes V-series Smartmodem 9600,

ATE1 V1 X1 S0=0S37=9N1Q0&Q5&K3&C1&S2^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,9600,60,CONNECT,1,1,

2400,CONNECT 2400^M,4800,CONNECT 4800^M,9600,CONNECT 9600^M,

### Hayes V-Series ULTRA Smartmodem 14400,

AT&F^M~ATW1 &C1&D2&S2 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### IBM High Speed Internal/PCMCIA Fax Modem,

AT&F^M~ATQ0W1X4 &C1&D2&K3 \\G0\\N3 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,38400,60,CONNECT,1,0,,,,,,,

### Intel 14.4ex Modem,

ATE1 V1 X4 S0=0 &D2&S1 \\G0\\J0\\N3\\Q3\\X1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Intel PCMCIA Faxmodem,

ATE1 V1 X4 S0=0 &D2&S1 \\G0\\J0\\N3\\Q3\\X1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,38400,60,CONNECT,1,0,,,,,,,

### Megahertz 14\,400 PCMCIA Data/Fax Modem,

AT&F^M~AT X4 W1 S0=0 &K3 %C3^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH^M~ATZ^M,~~~+++~~~ATS0=1^M,38400,60,CONNECT,1,0,,,,,,,

### Megahertz 14.4/14.4 FAX/Modem,

ATE1V1X4 S0=0 \\Q3\\J0\\N3\\X1\\V1 %C1^M,

ATDT,^M,~~~+++~~~ATH~ATZ^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Microcom QX/4232hs Modem,

ATE1 V1 X4 S0=0 &C1&D1 %C3 \\G0\\J0\\N6\\Q4^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Microdyne Access Modem (NACS),

ATE1 V1 &F \\Q3 &C1 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Motorola Codex 3220 Modem,

ATE1 V1 X4 &F&Y0^MAT &C1&S1 *FL3 S0=0 &W0^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Motorola Codex 326X Modem,

ATE1 V1 X4 &C1&S1 *FL3 S0=0^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Motorola UDS CELLect PCMCIA 14.4 Fax Mod,

AT&F^M AT S0=0 &D2&S1 \\Q3^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### MultiTech MT1432BR/MT932BR,

AT&F^M~AT S0=0 &C1&D0&E1&E4&E13&E15&Q0&S1^M~AT $BA0$MB14400$SB19200$R1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ^M~,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### MultiTech MultiModem II MT1432MU,

AT&F^M~AT S0=0 &E7&E13^M~AT $BA0$MB14400$SB19200$R1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ^M~,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### MultiTech MultiModem II MT1932BL/ZDX,

AT&F^M~AT S0=0 &E7&E13^M~AT $BA0$MB19200$SB19200$R1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ^M~,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### MultiTech MultiModem II MT2834BA,

AT&F^M~AT X4 S0=0 &E7&E13^M~AT $BA0$MB28800$SB115200$R1^M,

ATDT,^M,~~~+++~~~ATH0^M~ATZ^M~,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### MultiTech MultiModem224,

ATE1 &C1 V1 X1 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,2400,60,CONNECT,0,1,

300,CONNECT^M,1200,CONNECT 1200^M,2400,CONNECT 2400^M,

### MultiTech MultiModemV32,

ATE1V1X1S0=0 &E1&E4&E13&E15 $BA0$MB9600$SB19200$R1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Octocom OSI 8596 and 8596R Modem,

ATE1 V1 X4 S0=0 &C1&E1&I1&K2&R0 %H3%I1%S0 *E1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### PIICEON PCMCIA Dispatcher Data/Fax Modem,

AT&F^M~AT W1 &C1&K3&R1 %C3 \\N3^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### Practical Modem 2400 SA,

~~~ATE1 &C1&D1 V1 X1 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,2400,60,CONNECT,0,1,

300,CONNECT^M,1200,CONNECT 1200^M,2400,CONNECT 2400^M,

### Practical Peripherals 14400FX/14400FXSA,

ATE1 V1 X4 S0=0 &C1&K1&Q9&S2^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Practical Peripherals 9600 SA,

ATE1 V1 X4 S0=0 &C1&K3&Q9&S1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Practical PractiCARD 144 PCMCIA FaxModem,

AT&F^M~AT W1 &C1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,38400,60,CONNECT,1,0,,,,,,,

### Racal-Datacom ALM 3223,

AT&F^M~AT \\Q1\\X0 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Racal-Datacom ALM 3226 Cellular/Fax Mode,

AT&F^M~AT W1 S0=0 &C1^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Racal-Datacom ALM 3239,

AT&F^M~AT \\Q1\\X0 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Racal-Datacom RMD 3232 Modem,

AT&F^M~AT &C1 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Scout ScoutPlus V.32bis 14\,400 Modem,

AT&F^M~AT S0=0 &B2 *F3*M2*S1^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Spartan 9600 MNP5/V.42,

ATE1 V1 X1 S0=0 \\Q3 \\N6^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Telebit QBlazer Plus,

AT&F^M~AT X4 S0=0 S58=2 S68=255 S180=2 S190=1^M~AT S255=0 &W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Telebit T1600,

ATE1 &F9 S0=0 S51=5^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Telebit T2000 and TrailBlazer Plus,

ATE1 S0=0 S7=40 S50=254 S51=5 S52=1 S58=2 S66=1 S67=0 S68=2 S94=1 S95=2 S96=1 S131=1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Toshiba Noteworthy T144NW PCMCIA Modem,

AT&F^M~ATW1 S0=0 &C1&R0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH^M~ATZ^M,~~~+++~~~ATS0=1^M,38400,60,CONNECT,1,0,,,,,,,

### TWINCOM 144/DF Modem,

ATE1 V1 X4 S0=0 &C1&K3&Q9 \\N3 %C1%E1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### UDS Fast Talk V.32/42B,

~~~ATE1V1X4S0=0 &C1&S1 %B9600%C1 \\D1\\G\\J\\M1\\N7\\Q3\\R1\\T\\V\\X^M,

ATDT,^M,~~~+++~~~ATH~ATZ^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### UDS FasTalk II  14.4 V.32/42B,

~~~AT&F2^M~AT S0=0 &C1&D2 \\Q3\\V1\\X1^M,

ATDT,^M,~~~+++~~~ATH^M~~ATZ^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### UDS FastTalk 32bx,

~~~AT&F^M~ATH3 S0=0 &C1&D2&S1 \\Q3\\X1^M~AT&W^M,

ATDT,^M,~~~+++~~~ATH~ATZ^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### UDS V.3229/V.3229L Modem,

AT&F^M~AT &C1 \\Q3 \\V1 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Universal Data Systems V.32,

ATE1 V1 X4 S0=0 &C1&R&S1^M,

ATDT,^M,~~~+++~~~ATH^M,~~~+++~~~ATS0=1^M,9600,60,CONNECT,1,0,,,,,,,

### USRobotics COURIER HST,

ATE1 V1 X4 S0=0 &B1&H1&I0&R2^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,9600,60,CONNECT,1,0,,,,,,,

### USRobotics COURIER HST Dual Std. V.32bis,

ATE1 V1 X4 S0=0 S15=8 B0 &A3&B1&C1&H1&M4&N0&R2^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### USRobotics Dual Analog Modem V.32,

ATE1 V1 X4 S0=0 &B1&H1&I0&R2^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### USRobotics Sportster 14\,400,

AT&F &A3&B1&H1&K3&R2 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### USRobotics Sportster 9600/PC,

AT&F &A3&B1&H1&K3&R2 S0=0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### USRobotics WorldPort 14\,400 Modem,

AT&F^M~AT X7 S0=0 &B1&H1&R2^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,

### Xircom Creditcard Ethernet+Modem,

AT&F^M~AT^M~AT S95=47 W2 &Q9 \\X1^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,57600,60,CONNECT,1,0,,,,,,,

### ZOOM 14400 V.32bis FaxModem,

AT&F^M~ATW1 &C1&D2 S0=0^M~AT&W0^M,

ATDT,^M,~~~+++~~~ATH0^M,~~~+++~~~ATS0=1^M,19200,60,CONNECT,1,0,,,,,,,