." Modifications to the Valid 4.1BSD system to convert

." it so 4.2BSD running on S32s and SCALDsystem IVs.

."

."	Copyright  1984  Valid Logic Systems, Inc.

."

." jam 840829-0912

."

.TL

Valid Kernel Changes From 7.0 To 8.0

.AU

James A. Markevitch

.AI

September, 1984

.NH 1

Introduction

.PP

This paper describes the changes between the Valid 7.0 (4.1cBSD) and

the Valid 8.0 (4.2BSD) kernels.

It is assumed that the reader is familiar with 4.1cBSD, 4.2BSD, and 7.0 [1].

This paper only addresses

those modifications specific to the Valid kernel, not

those that were made between 4.1cBSD and 4.2BSD.

.NH 1

Bug Fixes

.PP

.NH 1

7.0 Compatibility

.PP

Unfortunately, some features which were present in 4.1cBSD were removed for

4.2BSD;

the signal mechanism changed and some ioctls were removed.

.NH 2

Ioctl Compatibility

.PP

In order for 7.0 programs like \fIstty\fR to work under 8.0 the following

ioctl calls had to be added to the 4.2BSD kernel:

TIOCSET, TIOCBIS, TIOCBIC, TIOCGET, TIOCCGET, and TIOCCSET.

These ioctls are conditionally compiled into the kernel using

\fIIOCTL4_1C\fR.

.NH 2

Signal Compatibility

.PP

The signal mechanism changed significantly from 4.1cBSD to 4.2BSD:

the \fIjobs(3)\fR calls became obsolete and were replaced by \fIsigvec\fR and

other system calls.

To guarantee that 7.0 programs would not have to be modified the 4.1cBSD

signal mechanism is implemented by the \fIintermedsendsig()\fR and \fIdorti()\fR

functions in the kernel, while the 4.2BSD signal mechanism is implemented

by the \fIsendsig()\fR and \fIsigcleanup()\fR functions.

The old signal mechanism (that is, the \fIsignal\fR system call) is still

implemented by \fIoldsendsig()\fR.

A new bit, SNUSIG, was added to \fIproc->p_flag\fR and functions in the same

manner as it did in 7.0.

The intermediate signal mechanism is conditionally compiled into the kernel

using \fIINTERMEDSIG\fR.

.NH 1

Performance Improvements

.PP

.NH 2

Context Register Use

.PP

In order to reduce the context switch time the hardware context register

is now used.

This reduces context switch times for better than 90% of the cases to

a few instructions (in \fIsureg()\fR).

The implementation allows up to 16 different processes and up to 15Mb of

total virtual program size to be kept in the hardware map.

Each of the two resources (contexts and segment maps) are re-used on an

LRU basis.

A complete description of the algorithm appears in a comment at the

beginning of \fIs32/context.c\fR.

The kernel has this feature conditionally compiled into it

with \fIUSE_CTXT\fR and also \fICTXT_STATS\fR and \fICTXT_DEBUG\fR to compile

in statistics which may be gathered with the \fIctxt\fR program, and to

compile in debugging messages, respectively.

.NH 2

Demand Paged Executable Files

.PP

The new kernel now supports demand paged executable files (standard to

4.2BSD) with type 0413 in the header.

These files are paged into memory on demand instead of being copied

in at \fIexec(2)\fR time.

The implementation, unfortunately, uses a 4K page size so that a

minimum of 4064 bytes (4K - sizeof(struct exec)) and an average

of 8160 bytes (4064 + rounding of text + rounding of data) are wasted in the

executable file on disk.

This could, forseeably, be reduced by changing the paging algorithm to use

512 byte (disk block size) granularity;

this may prove to be necessary on systems with virtually no local disk

resources since it would reduce network paging, but must be measured.

.PP

A number of user programs were modified in order to implement demand

paged executable files;

these were \fIld\fR, \fInm\fR, and \fIsize\fR.

In addition, a.out.h was modified to know about type 0413 files.

.NH 2

Pseudo-DMA On Serial Output

.PP

The \fIsc\fR and \fIoct\fR drivers now support pseudo-DMA on output

so that the output data rate is higher and the system overhead for it is lower.

In the implementation, a \fIpdma\fR structure is used to store the number

of characters in the current cblock being output in addition to a pointer

to the characters to be output.

When a transmitter interrupt occurs a piece of assembly code checks the

\fIpdma\fR buffer and outputs the next character without having to

call \fIscxint()\fR or \fIgetc()\fR, etc.

.NH 1

New Features

.PP

.NH 2

Autoconfiguration

.PP

The new kernel now works under the Berkeley 4BSD autoconfiguration

system.

This consists of the program \fIconfig\fR in \fI/etc\fR which reads

a configuration file and sets up a directory in which the kernel may

be compiled.

The standard system configuration is in \fIconf/ENG\fR and creates a

\fImakefile\fR and other files in the \fIENG\fR directory in the kernel

source tree.

.PP

The files \fIfiles\fR, \fIfiles.s32\fR, \fIdevices.s32\fR,

\fImakefile.s32\fR, and \fIdefines\fR in the \Iconf\fR directory

should be kept up to date in order for autoconfiguration to work.

.PP

In any event, the \fIident\fR in the configuration file should

be \fIGENERIC\fR in order for the correct root disk to be determined at

boot time.

.NH 1

References

.LP

[1]	James Markevitch.

.br

Features Specific to the Valid Virtual UNIX System

."

."

."

." Features done:

."

."	4.1c signals

."	4.1c ioctls

."	context register usage

."	demand paging

."	autoconfiguration

."	pseudo-DMA

."

." Features yet to do:

."

."	virtual console

."	text is rounded to segment boundaries

."	additions to ptrace for registers

."	u.u_usrtop

."

."	white kernel stuff

."	copy-on-write/copy-on-access

."	debugger

."	/dev/mbio

."	DFS

."	kernel stack problem

."	new user programs: ctxt, cow, config, symorder

."	user programs which changed: nm, size, ld, ps, strip(?), ranlib(?)

."	user programs to be recompiled: adb, umount

."	library functions which changed: nlist, umount, libl.a

."	new header file: a.out.h

."	new library functions: vmnlist, fsync

."	accounting tested

."	will quota be tested?