*********************************************************

*                                                       *

*           VGA - PROGRAMMING INFORMATION               *

*            FOR ADVANCED USERS Rev. 1.0                *

*                                                       *

*********************************************************

This document details information that software developers

will require to program the VGA video card in extended resolution modes.

Developers who require programming information about standard IBM or

Hercules video modes should refer to the Reference, located at the

end of this file.

The VGA video card incorporates a number of video display modes on one

board, care should be taken when programming to avoid video memory

conflict and/or programming to the wrong display mode. Damage to the

monitor may result if wrong mode parameters are programmed. Read through

this section and the example program before attempting to program

the VGA card.

EXTENDED RESOLUTION MODES

AH = 0  ;set video mode

AL =    MODE/             RESOLUTION      DIM/          START

TYPE                              COLOR         ADDRESS

---------------------------------------------------------------

0h      color/alpha       640x200         40x25/ BW     b800:0h

1h      color/alpha       640x200         40x25/ 16     b800:0h

2h      color/alpha       640x200         80x25/ BW     b800:0h

3h      color/alpha       640x200         40x25/ 16     b800:0h

0*      color/alpha       640x350         40x25/ BW     b800:0h

1*      color/alpha       640x350         40x25/ 16     b800:0h

2*      color/alpha       640x350         80x25/ BW     b800:0h

3*      color/alpha       640x350         80x25/ 16     b800:0h

0+      color/alpha       720x400         40x25/ BW     b800:0h

1+      color/alpha       720x400         40x25/ 16     b800:0h

2+      color/alpha       720x400         80x25/ BW     b800:0h

3+      color/alpha       720x400         80x25/ BW     b800:0h

4h      color/graphics    320x200         40x25/  4     b800:0h

5h      color/graphics    320x200         40x25/ BW     b800:0h

6h      color/graphics    320x200         80x25/ BW     b800:0h

7h      mono/alpha        720x350         80x25/ BW     b000:0h

0dh     color/graphics    320x200         40x25/ 16     a000:0h

0eh     color/graphics    640x200         80x25/ 16     a000:0h

0fh     mono/graphics     640x350         80x25/ BW     a000:0h

10h     color/graphics    640x350         80x25/ 16     a000:0h

11h     color/graphics    640x480         80x30/ BW     a000:0h

12h     color/graphics    640x480         80x30/ 16     a000:0h

13h     color/graphics    320x200         80x25/256     a000:0h

Enhanced Modes

23h     color/alpha      *1056x350        132x25/16     b800:0h

27h     mono/alpha       *1056x350        132x25/BW     b000:0h

33h     color/alpha      *1056x352        132x44/16     b800:0h

37h     mono/alpha       *1056x352        132x44/BW     b000:0h

53h     color/graphics    800x600         16            a000:0h

54h     color/graphics    800x600         16            a000:0h

61h     color/graphics    640x400         256           a000:0h

62h     color/graphics    640x480         256           a000:0h

63h     color/graphics    800x600         256           a000:0h

65h     color/graphics    1024x768        16            a000:0h

67h     color/graphics    1024x768        4             a000:0h

*132 column modes not available on NEC MultiSync 2A(and GS in analog mode).

The enhanced mode 61H, 62H, 63H, 65H, and 67H are supported

by the set mode (ah = 0) function only, the other BIOS functions will

not support these modes. It is up to the application programs or drivers

to implement those functions.

ENHANCED GRAPHICS MODES

The video modes described in this document are listed below by the

BIOS video mode number. We highly recommend that programmers select

the video mode through BIOS whenever possible.

Mode#   Resolution      Colors/         Memory          Monitors

(hex)                   Palette         Required        Supported

-----------------------------------------------------------------

54      800x600         16/256K         256K            3

61      640x400         256/256K        256K            1,2,3

62      640x480         256/256K        512K            1,2,3

63      800x600         256/256K        512K            3

65     1024x768         16/256K         512K            2,3

67     1024x768         4/16            256K            2,3

Monitors: 1) VGA, 2) IBM 8514, 3) Multisync

STATUS DETECTION

The VGA card is provided with some features which will allow software

to autodetect the presence of the VGA card, the amount of memory

installed, and the type of monitor for which it is configured.

Identifying the VGA card

c000:31   = `761295520'     - product signature

c000:40   = '31'            - Identifies product as

extended VGA card or compatible

c000:4c   =  xxh            - major BIOS revision number

c000:4d   =  yyh            - minor BIOS revision number

Monitor Configuration

The following assembler code segment can be used to determine for

which monitor the VGA card is currently configured.

cli

mov     dx,EXTENDED_REG

mov     al,DATA_I

out     dx,al

inc     dx

in      al,dx

sti

and     al,0fh

returns  al =   0       EGA monitor

1       PS/2 monochrome monitor

2       TTL monochrome monitor

3       PS/2 color monitor

4       RGB color monitor

5       Multisync monitor

7       PS/2 8514 monitor

9       NEC MultiSync 2A

b       NEC MultiSync 3D

d       NEC MultiSync XL

where

EXTENDED_REG = 01ceh

DATA_I = 0bbh

PROGRAMMING STANDARD EGA/VGA MODES WITHOUT BIOS

It is strongly recommended to set the video mode through BIOS whenever

possible. When this is not possible, such as when writing drivers

for non-DOS operating systems, direct manipulation of the CRT controller

parameters is necessary. A mechanism for accessing the parameter tables

is contained in the VGA BIOS. The following assembler code segment will

locate the start address of the parameter table for the desired mode.

mov     al,args

mov     ah,12h

mov     bx,5506h

mov	bp,0ffffh

int     10h

where args = mode desired in hex

If this routine returns BP = 0ffffh, that mode is not supported in

the current configuration. Else the parameter table is is ES:BP

The parameter table size and structure is identical to the IBM EGA/VGA.

The function of each register is also identical to IBM. Since the

VGA card uses hardware emulation, the standard parameters are used

regardless of the type of monitor installed. However, be cautioned

that the use of non-standard parameters may not produce the results that

you expect especially if the card is in emulation.

SELECTING MEMORY PLANES

Certain enhanced resolution graphics modes employ a multiple plane

memory organization. When one of these modes is invoked, the video

memory is organized into 64K planes at a0000h to affffh. VGA cards

with 256K memory have 4 planes and those with 512K have 8 pages available.

The following assembler code segment can be used to select the required

plane.

cli

mov     dx, EXTENDED_REG

mov     al,PLANE_SELECT

out     dx,al

inc     dl

in      al,dx

mov     ah,al

and     ah,PLANE_MASK

shl     ch,1

or      ah,ch

mov     al,PLANE_SELECT

dec     dl

out     dx,ax

sti

where           ch  =  required plane value

PLANE_SELECT = 0b2h

PLANE_MASK = 0e1h

MEMORY SIZE DETERMINATION

cli

mov	dx, EXTENDED_REG

mov	al,DATA_I

out	dx,al

inc	dx

in	al,dx

sti

and	al,020h

al = 0  ; 256K display memory installed

= 20h; 512K display memory installed

BIOS COMPATIBILITY

The VGA card is 100% BIOS compatible in all standard MDA, CGA, EGA and

VGA modes. The enhanced resolution graphics modes (modes  61h, 62h,

63h, 65h and 67h) do not support many of the BIOS functions.

The BIOS functions that are not supported are as follows:

Function  Description             Remark

AH =  1   Set cursor type         ; no cursor

AH =  2   Set cursor position

AH =  3   Read cursor position

AH =  4   Read light pen position ; no light pen port

AH =  5   Set active display page ; only one page available

AH =  6   Scroll page up

AH =  7   Scroll page down

AH =  8   Read character          ; no text mode

AH =  9   Write  character

AH =  A   Write character at specified page

AH =  B   Set CGA color palette

AH =  C   Write dot               ; use direct I/O to video memory

AH =  D   Read dot

AH =  E   Write TTY to active page

AH = 11   Character generator routines

AH = 13   Write string to specified page

All other BIOS routines should be supported including the mode switching

calls and the palette changing calls.

800x600 - 16 COLORS (Mode 54h)

Video Memory Organization

The memory organization is identical to 16 color EGA and VGA modes

and consists of 256K organized into 4 maps of 64K. One bit from each

of the 4 maps is used to compose each pel. The IBM EGA/VGA Map Mask

register is used to select any or all the maps to be updated.

A0000:0 -> ----------

|          |----------

|          |          |----------

|----------|          |          |---------- <===\=======\

| Reserved |----------|          |          |  60,000    |

----------| Reserved |----------|          |    |       |

Plane 0  ----------| Reserved |----------|<===/      64K

Plane 1  ----------| Reserved |            |

Plane 2   --------- <===========/

Plane 3

Video Data Format

The video data format is identical to 16 color EGA/VGA modes. Each

pel is represented by a 4 bit word. One bit from each of the 4 maps

are used to compose a pixel. Thus, each byte of address space contains

8 pels.

MSB*                                             LSB**

____ ____ ____ ____ ____ ____ ____ ____

Map 0 |    |    |    |    |    |    |    |    |  C0

|____|____|____|____|____|____|____|____|

____ ____ ____ ____ ____ ____ ____ ____

Map 1 |    |    |    |    |    |    |    |    |  C1

|____|____|____|____|____|____|____|____|

____ ____ ____ ____ ____ ____ ____ ____

Map 2 |    |    |    |    |    |    |    |    |  C2

|____|____|____|____|____|____|____|____|

____ ____ ____ ____ ____ ____ ____ ____

Map 3 |    |    |    |    |    |    |    |    |  C3

|____|____|____|____|____|____|____|____|

Pel     1     2    3    4    5    6    7    8

*Most Significant Bit

**Least Significant Bit

640x400 - 256 COLORS (Mode 61h)

Video Memory Configuration

This enhanced resolution mode uses a plane memory system unique

to this VGA card. The memory is organized into contiguous 64K

blocks located at A0000H to AFFFFH. The VGA card can be equipped

with either 512K or 256K of RAM to operate in this mode. Mode 61 uses

4 pages. The video data in this mode should only be updated during

monitor retrace periods.

A0000:0 -> ---------- <=========================================\

|          |----------                                |

|          |          |----------                     |

|          |          |          |---------- <==\=====|

|          |          |          |          |   |     |

----------|          |          |          |  64K    |

Page 0    ----------|          |          |   |   256,000

Page 1  ----------|          |   |     |

Page 2  ---------- <==/=====/

Page 3

Video Data Format

The video data format is identical to VGA mode 13h (320x200, 256 color).

Each pel is represented by 1 byte. Color selection and palette set

up is identical to VGA mode 13h.

640x480 - 256 COLORS (Mode 62h)

Video Memory Organization

This enhanced resolution modes uses a plane memory system unique

to this VGA card. The memory is organized into contiguous 64K blocks

located at A0000H to AFFFFH. The VGA card must be equipped with 512K

of RAM to operate in this mode. Mode 62 uses 5 pages.

A0000:0 -> ---------- <==============================================\

|          |---------                                      |

|          |         |---------                            |

|          |         |         |---------                  |

|          |         |         |         |--------- <=\    |

----------|         |         |         |         |  |    |

Page 0   ---------|         |         |         | 64K   |

Page 1  ---------|         |         |  | 307,200

Page 2  ---------|         |  |    |

Page 3  --------- <=/====/

Page 4

Video Data Format

The video data format is identical to VGA mode 13h (320x200, 256 color).

Each pel is represented by 1 byte. Color selection and palette set

up is identical to VGA mode 13h.

800x600 - 256 COLORS (Mode 63h)

Video Memory Organization

This enhanced resolution mode uses a plane memory system unique

to this VGA card. The memory is organized into contiguous 64K blocks

located at A0000H to AFFFFH. The VGA card must be equipped with 512K

of RAM to operate in this mode. Mode 63 uses 8 pages.

A0000:0 -> ---------- <==============================================\

|          |----------                                     |

|          |          |----------                          |

|          |          |          | \                       |

|          |          |          |  \                      |

----------|          |          |   \                     |

Page 0  ----------|          |    \                    |

Page 1  ----------       ---------- <=\  480,000

Page 3  \    |          |  |     |

\   |          | 64K    |

\  |          |  |     |

\ |          |  |     |

---------- <=/=====/

Page 7

Video Data Format

The video data format is identical to VGA mode 13h (320x200, 256 color).

Each pel is representecd by 1 byte. Color selection and palette set

up is identical to VGA mode 13h.

1024x768 - 16 COLORS (Mode 65h)

Video Memory Organization

This enhanced resolution modes uses a plane memory system unique

to this VGA card. The memory is organized into contiguous 64K blocks

located at A0000H to AFFFFH. The VGA card must be equipped with 512K

of RAM to operate in this mode. Mode 65 uses 8 pages.

A0000:0 -> ---------- <=============================================\

|          |----------                                    |

|          |          |----------                         |

|          |          |          | \                      |

|          |          |          |  \                     |

----------|          |          |   \                    |

Page 0   ----------|          |    \                   |

Page 1  ----------       ---------- <==\ 402,432

Page 2   \    |          |   |   |

\   |          |   |   |

\  |          |  64K  |

\ |          |   |   |

---------- <==/===/

Page 7

Video Data Format

The video data format is unique to this mode. Each pel is represented

by 4 bits with 2 pels per byte.

____________________________________________ High Order

|    |    |    |                              Pixel

__|_ __|_ __|_ __|_ ____ ____ ____ ____

|  H |  H |  H |  H |  L |  L |  L |  L |

|____|____|____|____|____|____|____|____|

|    |    |    |

|____|____|____|________ Low Order

Pixel

This mode has a different memory organization than a standard EGA

mode 10h. If users want to use colors other than the standard 16 colors,

users are required to re-program the external palettes (3C8h/3C9h).

Re-programming the internal palettes (3C0h) will not work correctly

because the color mapping in mode 65h is different from EGA mode 10h.

It is not recommended to change the internal palettes at all for mode

65h. The following table shows the mapping of colors and external

palettes.

Color  External Palette (Color Register)

0    00h,00h

1    01h,10h

2    02h,20h

3    03h,30h

4    04h,40h

5    05h,50h

6    06h,60h

7    07h,70h

8    08h,80h

9    09h,90h

A    0Ah,A0h

B    0Bh,B0h

C    0Ch,C0h

D    0Dh,D0h

E    0Eh,E0h

F    0Fh,F0h

For example, if user wants to change color 4 to high intensity

white, then the external palette 4 and 40h has to be re-programmed

with R=3FH, G=3Fh and B=3Fh.

1024x768 - 4 COLORS (Mode 67h)

Video Memory Organization

The memory organization is identical to 16 color EGA and VGA modes

and consists of 256K organized into 4maps of 64K. One bit from 2 of

the 4 planes is used to compose each pel. The IBM EGA/VGA Map Mask register

is used to select any or all the maps to be updated.

A0000:0 -> ----------

|          |----------

|          |          |----------

|----------|          |          |---------- <===\=====\

| Reserved |----------|          |          |    |     |

----------| Reserved |----------|          | 49,152   |

Plane 0  ----------| Reserved |----------|<== /     |

Plane 1  ----------| Reserved |         64K

Plane 2  ---------- <===/=====/

Plane 3

Video Data Format

This mode uses a unique data format where 2 bits are required per

pel. Plane 0 and 1 contain the data for odd numbered pels and Plane

2 and 3 containing the data of the even numbered pels.

This mode supports 1 of 4 colors sets. Selection of the color set

is done through bits 4 & 5 of the internal palette register (3c0h).

3c0 Bits     Color Values (C0, C1)

4 5        00      01      10      11

0 0        black   white   grey    bright white

0 1        black   green   red     yellow

1 0        black   cyan    red     white

1 1        black   cyan    magenta white

References

The following is a list of references which are a good source of

information for programmers who wish to write graphics drivers for

the EGA or VGA graphics standards.

Title: Personal System/2 Model 50 and 60 Technical Reference

Publisher: IBM

Date: April, 1987

Title: EGA/VGA A Programmer's Reference Guide

Author: Bradley Dyck Kliewer

Publisher: McGraw Hill

Date: 1988

Title: Programmer's Guide to IBM PC and PS/2 Video Systems

Author: Richard Wilton

Publisher: Microsoft Press

Date: 1987

Title: Programmer's Guide to the EGA/VGA

Author: George Sutty and Steve Blair

Publisher: Brady Books / Simon & Schuter, Inc.

Date: 1988

Title: Programmer's Guide to the EGA and VGA cards

Author: Richard F. Ferraro

Publisher: Addison-Wesley Publishing Co.

Date: July, 1988